EXP:4 7 SEGMENT DISPLAY USING AT89C51 USING PROTEUS

AIM:

To write an assembly language program for 7 Segment Display Using AT89C51 using Keil

and Proteus

SOFTWARE REQUIRED:

Proteus 8 software.

PROGRAM:

ORG 000H

UP:MOV P2,#0C0H

ACALL DELAY

MOV P2,#0F9H

ACALL DELAY

MOV P2,#0A4H

ACALL DELAY

MOV P2,#0Bон

ACALL DELAY

MOV P2,#99H

ACALL DELAY

MOV P2,#92H

ACALL DELAY

MOV P2,#82H

ACALL DELAY

MOV P2,#0F8H

ACALL DELAY

MOV P2, #80H

ACALL DELAY

MOV P2,#90H

ACALL DELAY

DELAY: MOV R5,#10

HI:MOV R4,#180

H2:MOV R3,#255

H3:DJNZ R3,H3

DJNZ R4.H2

DJNZ RS.HI

RET

END

RESULT:

Thus the program has been successfully verified and executed.